- 1b,3a,5b,11b,8b DHT 11 Sensor - Temperature and Humidity Readings

#include <Adafruit_Sensor.h> // include adafruit library

#include <DHT.h>

#include <DHT_U.h>

#define DHTPIN 32 // digital pin 32

#define DHTTYPE DHT11

DHT_Unified dht(DHTPIN, DHTTYPE);

void setup() {

Serial.begin(115200);

dht.begin();

}

void loop() {

sensors_event_t event; // It's a class under that give name event

dht.temperature().getEvent(&event); // dht is object,& address of object

Serial.print("Temperature: ");// print temp

Serial.print(event.temperature);// under event it displays temp

Serial.println("°C");

dht.humidity().getEvent(&event);

Serial.print("Relative Humidity: ");

Serial.print(event.relative_humidity);// serial is object

Serial.println("%");

delay(2000);

}

- 2b,4a,6b,10a OLED - Temperature and Humidity Readings

#include <Arduino.h>

#include <U8x8lib.h>

#ifdef U8X8_HAVE_HW_SPI

#include <SPI.h>

#endif

U8X8_SH1106_128X64_NONAME_HW_12C   u8x8(/*        reset=*/

U8X8_PIN_NONE);

#include <Adafruit_Sensor.h>

#include <DHT.h>

#include <DHT_U.h>

#define DHΤΡΙΝ 32

#define DHTTYPE DHT11

DHT_Unified dht(DHTPIN, DHTTYPE);

void setup(void)

{

u8x8.begin();

u8x8.setPowerSave(0);

u8x8.setFont(u8x8_font_chroma48medium8_r);

dht.begin();

}

void loop(void)

{

u8x8.setCursor (0,1);

sensors_event_t event;

// get temperature and print as float

dht.temperature().getEvent(&event);

u8x8.print("Temp:");

u8x8.print(event.temperature);

u8x8.print(" Cels");

// get humidity and print as integer

u8x8.setCursor(0,3);

dht.humidity().getEvent(&event);

u8x8.print("Hum:");

u8x8.print(event.relative_humidity);

u8x8.print("%");

delay(2000);

}

- 1a,7b,8a,11a LED/Buzzer - Turn On LED for 1 second after every 2 seconds

void setup() {

pinMode(2, OUTPUT); // Set GPIO2 as output

}

void loop() {

digitalWrite(2, HIGH); // Turn LED ON

delay(1000);

// Wait 1 second

digitalWrite(2, LOW); // Turn LED OFF

delay(2000);

// Wait 2 seconds

}

- 2a,9b Push Button/Digital Sensor (IR/LDR)

void setup() {

pinMode(18, OUTPUT); // Set GPIO18 as output for LED

pinMode(35, INPUT); // Set GPIO35 as input for Push button or Sensor

}

void loop() {

int val;

val = digitalRead(35); // Read push button or sensor state

if (val = HIGH) {

digitalWrite(18, HIGH); // Turn LED ON if button is pressed or object

} else {

digitalWrite(18, LOW); // Turn LED OFF otherwise

}

}

- 5a,10b Motor - Turn On when push button is pressed

// To interface motor using relay with Arduino/Raspberry

// Pi and write a program to 'turn ON' motor when

// push button is pressed.

void setup() {

// put your setup code here, to run once:

pinMode(13,OUTPUT);

pinMode(35,INPUT);

}

void loop() {

// put your main code here, to run repeatedly:

if(digitalRead(35))

{

digitalWrite(13,HIGH);

}

else

{

digitalWrite(13, LOW);

}

}

- 4b,9a Thingspeak Cloud (Retrieve)

#include <Adafruit_Sensor.h>

#include <DHT.h>

#include <DHT_U.h>

#define DHΤΡΙΝ 32

#define DHTTYPE DHT11

DHT_Unified dht(DHTPIN, DHTTYPE);

#include <WiFi.h>

#include "ThingSpeak.h"

const char* ssid = "Namratha";// your network SSID (name)

const char* password = "kruthika";// your network password

WiFiClient client;

unsigned long myChannelNumber = 3;

const char myWriteAPIKey = "M62TBVDF7LA9F203";

const char* readAPIKey = "P261LAD39SDRKP2X";

const long channelID=2456613;

const unsigned int firstReadFieldNumber = 1;

const unsigned int secondReadFieldNumber = 2;

const unsigned int switchField = 3; // Field number (1-8) to use to change status of device. Determines if data is read from ThingSpeak.

// Timer variables

unsigned long lastTime = 0;

unsigned long timerDelay = 30000;

float temperatureC;

float humidityP;

float temperatureR;

float humidityR;

void setup() {

Serial.begin(115200); //Initialize serial

dht.begin();

WiFi.mode(WIFI_STA);

ThingSpeak.begin(client); // Initialize ThingSpeak

}

void loop() {

if ((millis() - lastTime) > timerDelay) {

// Connect or reconnect to WiFi

if(WiFi.status() != WL_CONNECTED){

Serial.print("Attempting to connect");

while(WiFi.status() != WL_CONNECTED) {

WiFi.begin(ssid, password);

delay(5000);

}

Serial.println("\nConnected.");

}

sensors_event_t event;

dht.temperature().getEvent(&event);

temperatureC=event.temperature;

dht.humidity().getEvent(&event);

humidityP-event.relative_humidity;

Serial.print("Temperature: ");

Serial.print(temperatureC);

Serial.println("°C");

Serial.print("Relative Humidity: ");

Serial.print(humidityP);

Serial.println("%");

// set the fields with the values

ThingSpeak.setField(1,temperatureC);

ThingSpeak.setField(2,humidityP);

// Write to ThingSpeak. There are up to 8 fields in a channel, allowing you to store up to 8 different

// pieces of information in a channel. Here, we write to field 1.

int x = ThingSpeak.writeFields(myChannel Number, my WriteAPIKey);

if(x = 200) {

-Serial.println("Channel update successful.");

}

else{

Serial.println("Problem updating channel. HTTP error code " + String(x));

}

temperatureR=

ThingSpeak.readFloatField(channelID, firstReadFieldNumber, readAPIKey);

Serial.println(" Temperature read from ThingSpeak "+String(temperatureR));

humidityR=

ThingSpeak.readFloatField(channelID,secondReadFieldNumber,readAPIKey);

Serial.println(" Humidity read from ThingSpeak "+String(humidityR));

last Time = millis();

}

}

- 3b ThingSpeak Cloud (Upload)

#include <Adafruit_Sensor.h>

#include <DHT.h>

#include <DHT_U.h>

#define DHTΡΙΝ 32

#define DHTTYPE DHT11

DHT_Unified dht(DHTPIN, DHTTYPE);

#include <WiFi.h>

#include "ThingSpeak.h"

const char* ssid = "Namratha";// your network SSID (name)

const char* password = "kruthika";// your network password

WiFiClient client;

unsigned long myChannelNumber = 2;

const char *my WriteAPIKey = "KM0DAH7DAEWSL9P8";

// Timer variables

unsigned long lastTime = 0;

unsigned long timerDelay = 30000;

float temperatureC;

float humidityP;

void setup() {

Serial.begin(115200); //Initialize serial

dht.begin();

WiFi.mode(WIFI_STA);

ThingSpeak.begin(client); // Initialize ThingSpeak

}

void loop(){

if ((millis()-lastTime) > timerDelay) (

//Connect or reconnect to WiFi

if(WiFi.status() != WL_CONNECTED){

Serial.print("Attempting to connect");

while(WiFi.status() != WL_CONNECTED) {

WiFi.begin(ssid, password);

delay(5000);

Serial.println("\nConnected.");

sensors event_t event;

dht.temperature().getEvent(&event);

temperatureC=event.temperature;

dht.humidity().getEvent(&event);

humidityP-event.relative_humidity;

Serial.print("Temperature: ");

Serial.print(temperatureC);

Serial.println("°C");

Serial.print("Relative Humidity: ");

Serial.print(humidityP);

Serial.println("%");

// set the fields with the values

ThingSpeak.setField(1,temperatureC);

ThingSpeak.setField(2,humidityP);

// Write to ThingSpeak. There are up to 8 fields in a channel, allowing you to store up to 8 different

// pieces of information in a channel. Here, we write to field 1.

int x = ThingSpeak.writeFields(myChannelNumber, my WriteAPIKey):

if(x = 200) {

Serial.println("Channel update successful.");

}

else {

Serial.println("Problem updating channel. HTTP error code " + String(x));

}

lastTime = millis();

}

}

- 6a UDP

#include <WiFi.h>

#include <WiFiUdp.h>

#include <DHT.h>

// Replace with your network credentials

const char* ssid = "abcd";

const char* password = "ZXCV@098";

// Replace with your DHT sensor type and pin

#define DHT_PIN 32

#define DHT_TYPE DHT11

// UDP server port

#define UDP_PORT 5000

// Create an instance of the DHT sensor

DHT dht(DHT_PIN, DHT_TYPE);

WiFiUDP udp:

float humidity = 0.0;

void setup() {

Serial.begin(115200);

// Connect to Wi-Fi

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

Serial.println("Connecting to WiFi...");

}

Serial.println("Connected to WiFi");

// Print the ESP32 IP address

Serial.print("ESP32 IP address: ");

Serial.println(WiFi.localIP());

// Start the UDP server

udp.begin(UDP_PORT);

Serial.println("UDP server started");

// Initialize DHT sensor

dht.begin();

}

void loop() {

// Wait for incoming UDP packets

int packetSize = udp.parsePacket();

if (packetSize) {

// Read the incoming packet

char packetData[packetSize];

udp.read(packetData, packetSize);

String request = String(packetData);

if (request "get_humidity") {

// Read humidity from the DHT sensor

humidity dht.readHumidity();

// Send humidity data to the UDP client

udp.beginPacket(udp.remotelP(), udp.remotePort());

udp.printf("Humidity: %.2f%%", humidity);

udp.endPacket();

Serial.printf("Sent humidity data: %.2f%%\n", humidity);

}

}

}

- 7a TCP

#include <WiFi.h>

#include <WiFiClient.h>

#include <DHT.h>

// Replace with your network credentials

const char* ssid = "abcd";

const char* password = "ZXCV@098";

// Replace with your DHT sensor type and pin number

#define DHT_PIN 32

#define DHT_TYPE DHT11

DHT dht(DHT_PIN, DHT_TYPE);

WiFiServer server(8888); // TCP server port

void setup() {

Serial.begin(115200);

delay(10);

// Connect to Wi-Fi

Serial.println();

Serial.print("Connecting to ");

Serial.println(ssid);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

Serial.print(".");

Serial.println("");

Serial.println("WiFi connected");

Serial.print("IP address: ");

Serial.println(WiFi.localIP());

dht.begin();

server.begin();

}

void loop() {

WiFiClient client = server.available();

if (client) {

Serial.println("New client connected");

// Wait for data from client

while (client.connected()) {

if (client.available()) {

String request = client.readStringUntil('\r');

client.flush();

if (request.indexOf("/humidity") != -1) {

// Read humidity data from DHT sensor

float humidity dht.readHumidity();

if (isnan(humidity)) {

Serial.println("Failed to read humidity from DHT sensor");

client.println("Failed to read humidity from DHT sensor");

} else {

Serial.print("Humidity: ");

Serial.print(humidity);

Serial.println("%");

client.print("Humidity: ");

client.print(humidity);

client.println("%");

}

}

// Close the connection

client.stop();

Serial.println("Client disconnected");

}

}

}

}